GRANDFIRE CLASSIC SERIES BBQs NATURAL GAS CONVERSION INSTRUCTIONS

(Change out kits for GF26, GF32LB, GF38LB)

TOOLS NEEDED

1. 6mm Box Wrench

2. 2 Adjustable Wrenches

3. Phillips Screwdriver

4. Small Flathead Screwdriver

PARTS IDENTIFICATION:

All Models

1/2 NPT x 5/8 Flare

Fitting (1 piece)

All Models

NG Regulator

(1 piece)

Rear Burner Orifice

NG-Φ 1.78mm

GF26 (0)

GF32LB (1piece)

GF38LB (1piece)

Main Burner Orifice

NG-Φ 1.93mm

GF26 (2pcs)

GF32LB (3pcs)

GF38LB (4pcs)

SS Braided Hose

DI=12mm, Bayonet Quick Connection,

All Models (1 piece)

OPTIONAL PART:

SS Flex Tube

L=420mm

(1 piece)

WARNING!

To prevent personal injury or damage to the grill, a licensed, authorized, service technician or contractor must perform the installation. Installation must be done in accordance with the installation instructions included with this product.

GRANDFIRE CLASSIC GRILLS NG CONVERSION INSTRUCTIONS

| Step 1: (Change Main Burner Orifices) Remove all cooking grates and burner covers above. | Step 2: Unscrew the secure nut on the burner end. Lift burner end up and remove. |
| --- | --- |
| Step 3: Locate the valve with orifice in the hole on front basin. Use a 6mm box wrench to loosen LP orifice gently. | Step 4: Install NG orifices (Φ1.93mm) and gently tighten. DO NOT over tighten. |
| Step 5: Put main burner back and tighten the nut on the back. Make sure the orifice sits into the burner opening. | Step 6: (Change Rear Burner Orifice) Remove the rear burner window on the back. |
| Step 7: Take out the LP orifice a wrench(Size: 10mm) . Replace the NG orifice on and tighten it. | Step 8: Put the rear burner window back into place. |
| Step 9: (Change Regulator) Remove the LP regulator and fitting with two wrenches. | Step 10: Install NG regulator with hose on and connect hose to bayonet point. |
| Step 11: (Fire Test/Low Flame Setting) Refer to User Manual fire test and low flame setting procedures. |  |
| * The SS flex tube is optional for longer connections from BBQ grill to NG regulator. It is not a must installed part. |  |

Hauland PTY Limited. 
Website: www. Grandfirebbqs.com.au

info@grandfirebbqs.com.au